UNIVERSITY OF MUMBAI

DEPARTMENT OF COMPUTER SCIENCE

M.Sc. Computer Science – Semester IV

Robotics

JOURNAL

2023-2024

Seat No. 40423

UNIVERSITY OF MUMBAI

DEPARTMENT OF COMPUTER SCIENCE

CERTIFICATE

This is to certify that the work entered in this journal was done in the University Department of Computer Science laboratory by Mr..Koli Kalpesh Ananda Seat No. 40423 for the course of M.Sc. Computer Science - Semester IV (CBCS) (Revised) during the academic year 2023- 2024 in a satisfactory manner.

____________							 ____________

Subject In-charge                               			              Head of Department

__________________

External Examiner

INDEX

| Sr. No | Practical | Page No | Date | Sign |
| --- | --- | --- | --- | --- |
| 1 | Making a Raspberry Pi headless, and reaching it from the network using WiFi and SSH. | 1 |  |  |
| 2 | Using sftp upload files from PC. | 7 |  |  |
| 3 | Write Python code to test motors. | 18 |  |  |
| 4 | Develop Python code for testing the sensors. | 26 |  |  |
| 5 | Add the sensors to the Robot object and develop the line following behavior code | 28 |  |  |
| 6 | Add pan and tilt service to the robot object and test it. | 31 |  |  |
| 7 | Create an obstacle avoidance behavior for robot and test it. | 33 |  |  |
| 8 | Detect faces with Haar cascades. | 39 |  |  |

Practical No. 01

Aim: Making Raspberry pi headless, and reaching it from the network using wifi and SSH.

Implementation:

STEP I : Download raspberry pi imager for windows. And connect sd card to laptop.

STEP II : Select operating sysyem (raspberry pi OS 32-bit)

STEP III : Select Storage.

STEP IV : Click on Setting Icon . and set hostname. Click to enable SSH.

STEP V : Click to configure wireless LAN. And select country.

STEP VI : Click to save.

STEP VII : Click on write.

STEP VIII : Click on yes.

STEP IX : Verifying raspberry pi .

STEP X : Click on continue .

Step XI :  check the connection in your mobile hotspot of raspberrypi

STEP X:  On cmd promt execute these command :

- ping raspberrypi

- ssh admin@raspberrypi

Practical No 2

Aim: Using sftp upload files from PC.

Implementation :

Step 1: Intsall the Raspberry Pi Imager

Step 2: Set Host Name , enable SSH, Set Username and Password.

Step 3: Set SSID and Password of hotspot which is used .

Step 4: Connect Raspberry Pi WIFI and Laptop WIFI to Mobile Device.

Step 5: Open CMD and tyepe following command

- 1 ) ping raspberrypi or ping 162.168.207.244

- 2)  ssh admin@ raspberrypi or ssh admin@

162.168.207.244 And type password of Admin

Step:6 Download the FileZilla (Client)

Practical No: 03

Aim: Write python code to test Motors.

OUTPUT:

Give project name.

Select “create a schematic from the selected template”.

Select “Do not create a PCB layout”

Select “Create firmware project”.  and create firmware project in proteus and select Raspberry pi Family.

Click on Finish.

Go to the liabrary>pick devices>L293D

Select MOTOR

Edit terminal label as GPIO4

Edit terminal label as GPIO17

Select BUTTON

Rotate button in clockwise

Edit terminal label as GPIO18

Type code in proteus source code page

Start Simulation.

Practical No.4

Aim: Develop Python code for testing the sensors.

Step 1: Place the following component in TinkerCard.

Compontes:

- PIR Sensor

- Resistor

- Piezo

- Arduino Uno R3

- LED RGB

Step 2:Type the following code  int pirsensor = 0; void setup()

{

pinMode(2, INPUT);    pinMode(12,

OUTPUT);    pinMode(13, OUTPUT);

}   void loop()  {    pirsensor = digitalRead(2);    if (pirsensor == HIGH)

{

digitalWrite(13,HIGH);     tone(12,500,500);

}

digitalWrite(13,LOW)

}

Ouput:

Practical No: 5

Aim:- Add the sensors to the Robot object and develop the line-following behavior code.

Step 1: Download the following libraries

- L298 Motor Driver Library for Proteus

- Arduino UNO Library for Proteus V2.0

- Infrared Sensor Library for Proteus

Step 2: Extract this files and pest it in Proteus libraries folder and restart the Proteus .

Step 3: Download the Proteus IDE and type the following code  after that save it and Export Complied

Library

Step 4: Upload this HEX file in Arduino

Code:  void setup()

{

pinMode(2,INPUT); pinMode(3,INPUT); pinMode(10,OUTPUT); pinMode(11,OUTPUT);

pinMode(12,OUTPUT); pinMode(13,OUTPUT);

}

void loop(){

int v=digitalRead(2); int

s=digitalRead(3);

if(v==1 and s==1){

digitalWrite(13,1); digitalWrite(12,0); digitalWrite(11,1); digitalWrite(10,0);

}

if(v==1 and s==0){

digitalWrite(13,0); digitalWrite(12,1);

digitalWrite(11,0); digitalWrite(10,1); }

if(v==0 and s==1){

digitalWrite(13,1); digitalWrite(12,0); digitalWrite(11,0); digitalWrite(10,1);

}

if(v==0 and s==0){

digitalWrite(13,0); digitalWrite(12,1);

digitalWrite(11,0); digitalWrite(10,1);

}

}

Output:

Practical No. 6

Aim: Add pan and tilt service to the robot object and test it.

Take Arduino Uno R3 , Breadboard, two Servo motor and make connections:   Take two potentiometer and change resistance to 10: Join wires:

Code:

#include <Servo.h> int sensorValue = 0; int outputValue = 0; int sensorValue1 = 0; int outputValue1 = 0; Servo servo_9; Servo

servo_10; void setup()

{

pinMode(A0, INPUT);   servo_9.attach(9, 500,

2500);   pinMode(A1, INPUT);   servo_10.attach(10, 500, 2500); }  void loop()

{

sensorValue = analogRead(A0);    outputValue = map(sensorValue, 0, 1023, 0, 180);   servo_9.write(outputValue);    delay(10); // Delay a little bit to improve simulation performance    sensorValue1 = analogRead(A1);    outputValue1 = map(sensorValue1, 0, 1023, 0, 180);   servo_10.write(outputValue1);    delay(10); // Delay a little bit to improve simulation performance

}

Output:

Practical No. 7

Aim: Create an obstacle avoidance behavior for robot and test it.

Part A]

Take DC Motor and 9 Volt Battery, 2 Dc motors, Ardino UNO R3, 9V Battery and Breadboard small: Connect wires:

Output:

Take 2 more motors & connect wires:

Output:

Part B]

Take ultrasonic distance and connect :

Take LCD, Potentiometer and connect devices:

Take resistor and connect:

Write code:

//code for obstacle avoiding robot  #include <LiquidCrystal.h> LiquidCrystal lcd(8,9,10,11,12,13); long cm, duration;  const int echoPin = 7; const int trigPin = 6; const int lm1 = 2; const int lm2 = 3; const int rm3 = 4; const int rm4 = 5;  void setup()

{

pinMode(lm1, OUTPUT);   pinMode(lm2, OUTPUT);   pinMode(rm1, OUTPUT);   pinMode(rm2, OUTPUT);     pinMode(trigPin, OUTPUT);   pinMode(echoPin, INPUT);    Serial.begin(9600);  lcd.begin(16,2);

}  void loop()

{    digitalWrite(trigPin, LOW);   delayMicroseconds(2);   digitalWrite(trigPin, HIGH);   delayMicroseconds(5);   digitalWrite(trigPin, LOW);   duration = pulseIn(echoPin,HIGH);   //converting time into distance in centimetre     cm = duration*0.034/2;      if(cm < 20)

{      stop_bot();     delay(2000);       go_back();     delay(2000);       stop_again();     delay(1000);         go_left();     delay(1000);

}    else    {      go_straight();      delay(1000);

}

Serial.print("Distance:CM");

Serial.println(cm);

}

void go_straight()

{

lcd.setCursor(0,0); lcd.print("NOTHING AHEAD"); lcd.setCursor(0,1);  lcd.print("MOVING FORWARD");   digitalWrite(lm1,HIGH);   digitalWrite(lm2,LOW);   digitalWrite(rm1,HIGH);

digitalWrite(rm2,LOW);

}  void go_back()

{  lcd.clear(); lcd.setCursor(0,0);

lcd.print("TAKING REVERSE"); lcd.setCursor(0,1); lcd.print(cm);   digitalWrite(lm2,HIGH);   digitalWrite(lm1,LOW);   digitalWrite(rm2,HIGH);

digitalWrite(rm1,LOW);

}

void stop_bot()  { lcd.clear(); lcd.setCursor(0,0);

lcd.print("SOMETHING AHEAD");

lcd.setCursor(0,1); lcd.print("STOP!");   digitalWrite(lm1,LOW);   digitalWrite(lm2,LOW);   digitalWrite(rm1,LOW);

digitalWrite(rm2,LOW);

}  void stop_again()  { lcd.clear(); lcd.setCursor(0,0); lcd.print("BREAK FOR TURN");   digitalWrite(lm1,LOW);   digitalWrite(lm2,LOW);   digitalWrite(rm1,LOW);

digitalWrite(rm2,LOW);

}  void go_left()

{ lcd.clear(); lcd.setCursor(0,0); lcd.print("TURNING LEFT"); lcd.setCursor(0,1);   lcd.print(cm);     digitalWrite(lm1,LOW);   digitalWrite(lm2,LOW);   digitalWrite(rm1,HIGH);

digitalWrite(rm2,LOW);

}  void go_right()

{ lcd.clear(); lcd.setCursor(0,0); lcd.print("TURNING RIGHT");

lcd.setCursor(0,1);   lcd.print(cm);     digitalWrite(lm1,HIGH);   digitalWrite(lm2,LOW);   digitalWrite(rm1,LOW);

digitalWrite(rm2,LOW);

}

OUTPUT:

Practical No: 8

Aim: Detect faces with haar cascades.

CODE:

# Importing OpenCV package import cv2

# Reading the image img = cv2.imread('test.jpg')

# Converting image to grayscale  gray_img = cv2.cvtColor(img, cv2.COLOR_BGR2GRAY)

# Loading the required haar-cascade xml classifier file haar_cascade = cv2.CascadeClassifier(cv2.data.haarcascades

+ "haarcascade_frontalface_default.xml")  eye_cascade = cv2.CascadeClassifier(cv2.data.haarcascades + 'haarcascade_eye.xml')

# Applying the face detection method on the grayscale image faces_rect = haar_cascade.detectMultiScale(gray_img, 1.3, 5) eyes = eye_cascade.detectMultiScale(gray_img)

# Iterating through rectangles of detected faces for (x, y, w, h) in faces_rect:      cv2.rectangle(img, (x, y), (x+w, y+h), (0, 255, 0), 2) for (ex,ey,ew,eh) in eyes:       cv2.rectangle(img,(ex,ey),(ex+ew,ey+eh),(0,255,0),2)

cv2.imshow('Detected faces', img)

cv2.waitKey(0)

OUTPUT: